Ultrarotdurchlässigkeit

For the measurements a Temperature of 75°C was selected. Room temperature was at around 20°C. After inserting the filters x fraction Q of the Intensity without filter remained.:

\begin{align}

Q = \frac{U_x}{U_{no filter}}

\end{align}

No filter: 100,0% +- 4,1

Nickel plated: 0,00% +- 0,38

Window glass: 2,06% +-0,44

Silicon (front mirror): 46,4%+-2,0

Silicon (backside mirror): 46,7% +- 2,1

Silicon (averaged): 46,6%+-1,7

NaCl: 78,6 % +- 3,3

Infrared Glass: 1,39 % +-0,42

The Error is calculated from The Agilents 2,5% +0,8mV DC Error in the 100mV sensitivity range (Anleitung zitieren) and gaussian propagation of uncertainty. The measurement with a nickel plated filter was used as zero. (For silicon the average of both directions was used). The wavelength peak $\lambda_{Peak}$ of Planck’s thermic radiation with which the filters were hit can be calculated with the Wien’s displacement law eq.(\ref{Wien}).

\begin{align}

\lambda_{Peak} = \frac {b}{T} \label{Wien}

\end{align}

$b \approx 2,897771955 \cdot 10^{-3} \,\text{m}\cdot\text{K}$ is \textit{Wien’s displacement constant} and $T$ is the absolute temperature in Kelvin. From this we can calculate the wavelenghs around which the radiation is ?centered? at room Temperature and at the Temperature of \textit{Leslie’s cube}.

\begin{align}

\lambda_{room} &= (9.885\pm 0.034)\,\mu m \\

\lambda_{Leslie} &=(8.323\pm 0.024)\,\mu m

\end{align}

Bothe these wavelengths lie firmly inside the infrared region of the em-spektrum. All the radiation hitting the thermic element in this experiment should have a wavelength inbetween these two values.

!Diskussion und Vergleich!